Work Instructions Laboratory Calculations

These work instructions are for help when performing laboratory calculations using the Reed Muench method or Spearman Karber.

- Reed Muench Background

- Most titer calculations for poultry products are based on the Reed Muench method. This work instruction is to help with the calculation of 50 percent endpoint using the Reed Muench method and other helpful information. When calculations are based off of tissue culture testing the result would be Tissue Culture Infective Dose50 (TCID50/dose), when based off of egg inoculation the result would be Egg Infective Dose50 (EID50/dose)

- CVB Poultry Quality Management Documents vs. Firm Outline of Production/Special Outlines

- CVB Supplemental Assay Methods (SAMs) and Protocols (PROs) pertaining to egg testing are written to dilute the product in such a manner that final calculation is considered to be EID50/dose. They include additional dilution and amount inoculated to a per dose read-out. No further calculation is needed after the basic 50% endpoint. See Appendix 1 for breakdown of dilution.

- Firm Outline of Productions (OP) and/or Special Outlines (SO) may require additional calculation to reach the EID50 “per dose” titer. See Appendix 2 for breakdown of dilution.

- Importance should be placed on tracking nomenclature as this will help with knowing what calculation is needed. Example: egg infected dose 50/mL (EID50/mL)

- Reed Muench (50% endpoint) calculation

- Table 1 with example.

| Dilution | # of negative | # of positive | Negative cumulative Add top down | Positive cumulative Add bottom up | #pos cumulative/total pos+neg cumulative | % positive |
| --- | --- | --- | --- | --- | --- | --- |
| 10-1 | 0 | 5 | 0 | 14 | 14/14 | 100 |
| 10-2 | 0 | 5 | 0 | 9 | 9/9 | 100 |
| 10-3 | 2 | 3 | 2 | 4 | 4/6 | 67** |
| 10-4 | 4 | 1 | 6 | 1 | 1/7 | 14*** |
| 10-5 | 5 | 0 | 11 | 0 | 0/11 | 0 |

  - Dilution; write in the dilution inoculated.

(In this example; 10-1 thru 10-5 were inoculated)

  - # of negative; write in the number of negative eggs for each dilution.

  - # of positive; write in the number of positive eggs for each dilution.

  - Negative cumulative; starting at the top of the “# of negative” column, add going down.

  - Positive cumulative; starting at the bottom of the “# of positive” column, add going up.

  - # pos divided by total pos-neg; for each dilution, write the number in the positive cumulative column divided by the positive cumulative + negative cumulative.

  - % positive; calculate # positive divided by positive+negative then multiply by 100.

- What dilution is at least 50% positive? This dilution is the start of the titer.

(In this example; 10-3 is 67% and 10-4 is 14%. 3 is the first number in your titer.)

  - % positive at least 50% or above (67) minus 50

(Example; 67 – 50 = 17)

  - % positive at least 50% or above (67) minus % below 50 (14)

(Example; 67 – 14 = 53)

  - Result from “a” divided by result of “b” = the decimal point of your titer. (Example; 17/53 = 0.32)

  - Example; 10-3 (3) + 0.32 = the titer 3.32 Egg Infected Dose50 (EID50).

- Additional calculation if needed to reach a “per dose” titer.

  - As stated in II.B. It is important to keep proper nomenclature when writing out the titer. How the sample was initially diluted and how much was inoculated must be taken into account during these final calculations.

- Spearman Karber Background

A. Spearman Karber is based on 50 percent endpoint using the Spearman Karber method and final result would be noted as Infective Dose50 (ID50/dose). When calculations are based off of tissue culture testing the result would be Tissue Culture Infective Dose50 (TCID50/dose), when based off of egg inoculation the result would be Egg Infective Dose50 (EID50/dose)

Spearman-Kärber as modified by Finney. Test Vaccine titer = (X - d/2 + [d • S])

Test Vaccine titer = (X - d/2 + [d • S]) where:

X = Log10 of highest dilution with 100% positive

d = Log10 of dilution factor (In most cases tenfold dilution =1, but need to make sure this is correct)

S = Sum of proportion of CPE positive (X + any positives in subsequent dilutions/number of wells inoculated per dilution)

Example: (Tenfold dilutions made and 0.2mL inoculated per well.)

100  dilution of the Test Vaccine = 5 of 5 wells CPE Pos

10-1 dilution of the Test Vaccine = 5 of 5 wells CPE Pos

10-2 dilution of the Test Vaccine = 3 of 5 wells CPE Pos

10-3 dilution of the Test Vaccine = 0 of 5 wells CPE Pos

In this example: (X - d/2 + [d • S]):

X = 1

d = 1

S = 1.6 [(10-1 pos 5/5) +(10-2 pos 3/5)+(10-3 pos 0/5)] =  8/5 = 1.6

(X - d/2 + [d • S]) = 1 – 1/2 + [1 • 1.6]) = 2.1 ID50/0.2mL

Adjust the titer to the recommended Test Vaccine dose as follows:

1.	 Divide the Test Vaccine Dose by the Inoculation Dose

Test Vaccine Dose = manufacturer’s recommended vaccination dose (for this test vaccine, the recommended dose is 2 mL)

Inoculation Dose = amount of diluted Test Vaccine added to each well of the Test Plate (for this test vaccine, the inoculation dose is 0.2 mL)

2 mL dose

0.2 mL	= 10

2. 	Calculate log10 of value in A and add it to the Test Vaccine titer as illustrated below:

Log of 10 = 1.0

Test Vaccine titer	= 2.1 + 1.0 = 3.1

Therefore the titer of the Test Vaccine is 103.1 TCID50/1 mL.

Table 2. This information is to help in cases of wrong dilutions or inoculation to aid in the final calculations.

| Numbers | Logs |
| --- | --- |
| Multiply by 2 | Add 0.3 logs |
| Divide by 2 | Subtract 0.3 logs |
| Multiply by 3 | Add 0.5 logs |
| Divide by 3 | Subtract 0.5 logs |
| Multiply by 5 | Add 0.7 logs |
| Divide by 5 | Subtract 0.7 logs |
| Multiply by 10 | Add 1.0 logs |
| Divide by 10 | Subtract 1.0 logs |

- An example of the use of this information would be 200µl was inoculated but should have been 100µl. This is 2 times the required amount; therefore, 0.3 would be subtracted from the final titer.

Appendix

1: 	CVB dilutions of product to reach a direct read-out of “per dose”.

Dose = 0.03mL

30 mL diluent per 1,000 doses = 33.3 doses/mL

1.5mL + 3.5mL = 1.5mL contains 50 doses + 3.5mL diluent = 50 doses in 5mL

1 mL would contain 10 doses

Make ten-fold dilutions and inoculate 0.1mL/egg = 1 dose/egg

= direct read-out per dose

2: 	Firm’s dilutions of product with additional calculation to reach “per dose”.

Dose = 0.03mL

30 mL diluent per 1,000 doses = 33.3 doses/mL = 100

Make ten-fold dilutions and inoculate 0.2mL/egg = 6.7 doses/egg

The log conversion to reach the “per dose” titer is an additional two steps

1 – Convert to titer/mL – since 0.2 was inoculated, would multiply by 5 or according to

Table 2, add 0.7 logs.

2 – Convert to titer/0.03mL – first divide 1 by 10 to equal 0.1 = subtract 1.0

Second divide by 1.0 by 3 to equal 0.03 = subtract 0.5

= final titer on a “per dose” basis